Here’s some ways to manually raise the lift. You will have to get someone else to do this procedure.

Kind regards,

Mike

***

Here are the instructions for manually raising the lift.

There are two ways to manually raise the lift: by all means, try the first one as it is quite a bit easier based on the lift being in the contracted position. Image 4208 shows the bolt placement:

***************

Pulling the motor assembly

- Remove the motor cover (image 2429)

- Find 4 allen bolts holding the motor assembly to the lift plate – the bolts are in the 4 corners of the motor assembly (image 4208)

- After removing the bolts, pull the motor assembly out perpendicular to the lift and the lift will freewheel – be careful and prop up the lift/TV so it does not crash down! Best to have someone hold the lift while you pull out the motor assembly, then raise it all up and place that motor assembly back in. Just to ensure it remains put, put one of the bolts back in to hold the motor assembly in place.

************

Pulling the motor gear – this is more difficult and should only be done if you can’t remove the motor.

Remove motor cover (image 2429)

Use a screwdriver to remove the small gear washer (pry it out sideways – there is an opening in this washer Image 2431)

Pull either the large or small gear off the gear spline (see image 2432). You only have to remove one gear and one may be easier to remove (image 2563).

The lift will now free-wheel – you can either simply hoist up the lift or use vice grips (Image 2433) to wheel it up . When the lift is to desired spot, put the small gear back on the gear spline and the lift will stay in place.